Summary (Analog): Usually there won’t be an “explosion” on the street (conflicts during multithreaded execution), so pedestrians are allowed to move quickly (threads execute concurrently). Once an “explosion” is detected (by cache-coherence–based detection), everyone is sent back home (roll-back for recovery), and the area is blocked off with police tape (fall back to lock-based execution).

Weakness:

1) Resource constraints can cause failures. For example, uncached accesses or events can force speculation to fail and cannot continue in the speculative mode.

2) If conflicts are frequent, rollbacks happen often. The overhead of moving/handling speculative state can be high, and SLE may be slower than traditional locking.

Potential Avenue:

1) Handle more untrackable events (e.g. system calls, I/O, uncached accesses).

2) Better ways to assign confidence to locks to reduce pathological rollbacks.

A Sample Exam Question: Compared to using the reorder buffer (ROB) to buffer speculative register state, what is a key advantage of using a register checkpoint in SLE?

A. Using a checkpoint removes the critical-section size (in terms of dynamic instructions) limitation.

B. It is simpler to implement.

C. It eliminates the need to add an access bit to each cache block.

D. It enables tracking of uncached accesses and system-call effects. (It doesn’t need to worry about this case.)

Answer: A

Explanation for other choices:

B: Not necessarily; checkpointing may require additional hardware/complexity.

C: False; checkpointing does not eliminate the need for access bits.

D: False; uncached accesses and system-call effects are still not trackable in general and require conservative fallback.