Summary (Analog): Imagine a large storage locker outside a classroom that would normally need to reserve one full compartment for each of two students. However, if a copy of one student’s belongings is already stored in a small locker inside the classroom, then the large locker does not need to keep both students’ items in full. Instead, it can store only a specially encoded combination of the two. Later, when one student’s belongings are needed, we can reconstruct them by combining that encoded version with the copy already available in the small locker.

Weakness: (1) Its benefit depends heavily on workload characteristics. For example, if many cache lines are in the M state, or if many shared lines are non-unique, the compression ratio can become much lower. (2) The design relies on structural constraints that may reduce its generality. For example, it requires the minimum sharer invariant, and in the current design each cache line can have only one XOR partner.

Potential Avenue: To extend XOR Cache to a wider range of system environments. For example, evaluate whether XOR Cache remains practical and effective in larger-scale and more diverse cache hierarchies, or even in heterogeneous CPU–GPU memory systems.

A Sample Exam Question: Why must an XORed cache line usually be unXORed before it is upgraded to the Modified state?

A. Because after a write, the compressed copy in the LLC may become stale and the data may no longer be recoverable.

B. Because a directory cannot maintain a sharer list in the Modified state.

C. Because XOR Cache does not support writes.

D. Because XOR Cache does not support coherence.

Answer: A