Analog: If employees do not have clear responsibilities, a manager must constantly monitor each employee (traditional coherence continuously tracks sharers and issue invalidations). In DeNovo, if employees have clear responsibilities, such a strict manager is no longer necessary (each cache can simply perform self-invalidation at the end of a phase based on the write-region information provided by the program).

Weakness: (1) As the paper itself notes, in a real system the region information should be conveyed by the compiler. However, in this work, object-region information is manually provided through APIs, and some communication-space information is also inserted manually. Thus, the path from language-compiler-programmer annotations to a truly deployable system is not yet fully closed. (2) If the software or compiler is not able to provide imprecise region information, self-invalidation may become more conservative, causing unnecessary invalidations and reducing the performance benefit.

Potential Avenue: (1) replace these manual API annotations with more complete compiler automation, such as automatically generating region information and self-invalidation insertion logic. (2) study how to reduce overly conservative invalidations when region information is imprecise.

A Sample Exam Question: What is the key change in DeNovo compared with traditional directory-based MESI?

A. It tracks a larger sharer list.

B. It uses software-supported self-invalidation to remove the need for directory sharer tracking and write invalidation.

C. It allows data races to reduce synchronization overhead.

D. The program/compiler explicitly provides region information so caches can self-invalidate at phase boundaries.

Answer: B, D