Summary (Analog): A conventional NoC is like a train station with waiting rooms, where each router contains buffers. BLESS, in contrast, is like an intersection where cars are not allowed to stop: once a flit arrives at a router, it can’t wait and must be forwarded to some output port. If it can’t take the most desirable direction, it is immediately turned onto another path and loops back later through deflection.

Weakness: (1) BLESS is highly load-dependent. When network utilization is high and deflections become frequent, its saturation throughput is lower than that of a buffered network. (2) The design doesn’t provide sufficient support for more realistic scenarios, such as QoS, multiple traffic classes, congestion awareness, and fault tolerance. (3) Although router input buffers are eliminated, receiver-side buffering requirements and packet reordering complexity increase, so overall system complexity is not truly reduced.

Potential Avenue: (1) Extend the design to more complex and realistic NoC settings. (2) Study how to reduce receiver-side buffering and packet reordering complexity while preserving the energy benefits of bufferless routers. My thought is to keep following flits on the same path after routing the head flit, and only allow a packet to split when contention becomes severe.

A Sample Exam Question: Why can BLESS reduce router latency?

A. Deflection reduces path length.

B. It eliminates buffer writes and simplifies buffer management.

C. It removes the need for Virtual Channel allocation.

D. The receiver no longer needs any buffering.

Answer: B, C